SLASH SPEC

Oval Slash rules:

BOTH LCG AND HCG chassis are legal!!!!!! Minimum weight: 5lb 8oz. (any added weight must be inside the main chassis area)

must use stock traxxas slash 2wd chassis and components unmodified, unless otherwise specified. No modifying, replacing, or removing any components unless otherwise specified.

All suspension components must be installed in the proper position and location on the car from the factory (Example: Left caster block on the left, right caster block on the right, suspension arms right side up and on the proper corner of the chassis, etc.) must use grey or black plastic hexs only.

Any color Traxxas Slash aluminum shock caps, steering bell cranks (#3743), caster blocks (#3632), steering blocks (#3636), camber links and rear axle carriers (#3652) are allowed. Other manufacturer’s comparable parts are allowed for use. They must be exact replacement parts and are limited to the identical Traxxas parts listed above (Example: No aluminum shock towers, transmission cases, suspension arms and so forth). Rear axle carriers (bearing carriers) that add additional toe in or out are not allowed.

RPM rear carriers (#80382), front steering blocks (380372), caster blocks (#80712), and front (#80952, #80953, and #80955) and rear (#81002, #81003, and #81005) bumpers will be allowed.

If running the stock front bumper, the hoop and ears on the front bumper can be cut off, however, the main area of the bumper must remain intact. you may run the bumper kit that comes with the mod or latemodel bodies.

Transmitter/Receiver and servo may be replaced. (Servo saver will not be a tech item) Stock transmissions only!!!

Transmission:

Locking or modifying the differential in any way will not be allowed. The differential must have normal diff action when rotating one rear tire against the normal compression (resistance) of the motor. ALL METAL GEARS ONLY, NO PLASTIC TRANNY GEARS!!!!!! transmission has to be ran as it comes with the exception of a hot racing diff. To not allow diff fluid to leak.

No Sway Bars.

Any additional weight must be added to the main chassis plate area. Adding weight to nerf bars, bumpers, headers, suspension arms, or any component not within the main chassis area will not be allowed. The ESC, transponder, and receiver can be mounted anywhere inside the main chassis plate area only!

MOTOR/ESC:

Must use a stock XL-5 ESC and unaltered Titan 12T 550 brushed motor or hobbywing justock 13.5 zero timing combo ONLY! Tuning, tweaking, or modifying the motor in any way will not be allowed! Any motor can be claimed for $70 or a new unopened motor (failure to give up motor will result in DQ for the one being claimed)

The stock (bullet) connectors from the motor to the ESC can be removed, direct soldering from esc to motor is Legal.

No limit on gearing. However, the Traxxas spur gears (#4683, #4686, and #4690) must be used. No slipper eliminators are allowed.

Battery:

Any 2 cell Li-Po battery

Battery must be mounted in the stock center tray location, but can be moved forward or back, as long as it stays within the stock boundaries of the battery tray.

Any style battery plug may be used for the ESC battery connection. However, indirect soldering to the battery will not be allowed.

Tires/Wheels/Inserts:

Any 2.2/3.0 short course wheel will be allowed.

Must use Traxxas Slash tires. Choice of Spec Slash, Kumho Venture, and BF Goodrich treads.

S-1 compound tires ARE legal! One foam insert per tire.

any foam insert is legal.

Shocks/Springs:

Stock Traxxas Slash progressive springs ONLY!!! This means the white factory springs that come on a 2wd Traxxas Slash truck (front spring #5857 and rear #5858).

Springs cannot be cut or altered from factory specs in any way.

The shock oil or shock travel/length will not be mandated.

Two stage pistons or any modification to the shock piston to tune the compression and/or rebound will not be allowed. Must use the Traxxas Slash pistons.

Bodywork: Any sc type body